Virgo BIOS Update Process

This is the manual for updating BIOS on Virgo system. Please check current system BIOS version is VIRH1030 or later. Here are the update procedures

DOS:

1. Copy VIHH1060.bin into dos folder

2. Copy dos folder to USB stick or HDD

3. Enter to DOS folder and execute the below command

flash.bat

4. Reboot if complete the updated

Linux:

- Copy VIHH1060.bin into

linux/ InsydeH2OFFT_x86_LINUX64_200.02.00.02 folder

- Copy linux folder to USB stick or HDD

- Enter to linux folder and execute the below command

./flash.sh

- Reboot if complete the updated

EFI:

- Copy VIHH1060.bin into efi folder

- Copy efi folder to USB stick or HDD

- Boot into internal shell enters the usb efi folder and executes the below command Bios.nsh

- Reboot if complete the updated