| Revision | ECN | Description | Date (dd/mm/yyyy) | Approval |
| --- | --- | --- | --- | --- |
| A |  | FIRST RELEASE | 2013/12/30 |  |
|  |  | Add windows update, SOCFlash update | 2015/11/30 |  |
|  |  | Modify execute command | 2016/02/19 |  |
|  |  | Modify execute command | 2016/03/29 |  |
|  |  | Remove windows update information Remove Linux32 update information Add BMC WEB GUI remote update information Add BMC Yafuflash remote update information Add BIOS WEB GUI remote update information Add BIOS Yafuflash remote update information | 2021/09/07 |  |

1	Scope	3

2	Reference Docs	3

3	Requirement	4

4	Update guide	5

4.1	DOS	5

4.2	LINUX	6

4.3	Remote update (WEB GUI)	7

4.4	Remote update (Yafuflash)	12

5	SOC Flash update	13

6	Remote update BIOS firmware via BMC guide	13

6.1	Yafuflash	13

6.2	WEB GUI	14

# Scope

The scope of this document is to update BMC firmware.

# Reference Docs

# Requirement

Support OS & browser list.

DOS:    MS-DOS or Free-DOS

LINUX:  Redhat 6.2, fedora 17, CentOS 6.2

Brower:  I.E 6.0 or later version, Firefox 24.0 or later version, Chrome 31.0 or later version.

# Update guide

## DOS

1. Update BMC firmware.

a. Boot to DOS (MS-DOS or Free DOS is workable).

b. Enter BMC firmware directory [xxxxxxxyyzzzz];

xxxxxxx: project name ;

yy: system SKU number ;

zzzz: BMC firmware version .

c. Execute “A.BAT “ to update BMC firmware

DOS update config will be clear.

Example:

A:>cd HA401VG012033

A:\ HA401VG012033>A.BAT

PS: This is just an example, " HA401VG012033" is the BMC firmware file name.

Please instead " HA401VG012033" to your BMC firmware directory name.

d. If BMC is not responding, you can use “A.BAT fix” to update.

2. After update BMC firmware, please power off and then power on system.

Note:

1. Do not use EMM386 in the DOS environment when update firmware or you will get an exception failure.

2. In some critical conditions, after update BMC firmware, you might need to unplug the AC power cord for 5 seconds and then plug in the AC power cord to reset BMC, then the updated new function can work properly.

3. It will do full firmware update. (Config will be clear.)

## LINUX

1. Update BMC firmware.

a. Boot to Linux.

b. Enter BMC firmware directory [xxxxxxxyyzzzz];

xxxxxxx: project name ;

yy: system SKU number ;

zzzz: BMC firmware version.   .

c. Execute a.sh shell script to update BMC firmware.

Example:

Linux x64

#cd HA401VG012033/Linux

Linux#./a.sh

PS: This is just an example, " HA401VG012033" is the BMC firmware file name.

Please instead " HA401VG012033" to your BMC firmware directory name.

d. If you don’t want to save config and want to use ”YafuFlash” to update,  you can use “a.sh noconfig”

e. If you don’t want to save config, you can use “a.sh soc” to update.

f. If BMC is not responding, you can use “a.sh fix” to update.

P.S.: config will be clear.

2. After update BMC firmware, please power off and then power on system.

Note:

- Please change your account to root, or you will get an exception failure.

- In some critical conditions, after update BMC firmware, you might need to unplug the AC power cord for 5 seconds and then plug in the AC power cord to reset BMC, and then the new function can work properly.

- Fedora 12 x86, Fedora 17 x64, and Redhat 6.2 x64 are workable.

## Remote update (WEB GUI)

Log into the BMC web page

Click the “Maintenance”, and then Click the “Firmware Update”

Select firmware ima file and click “Start firmware update”.

Select “Preserve all Configuration” to preserve all setting.

Or click “Edit Preserve Configuration” link to select independent setting which you would like to preserve.

Return to Firmware Update page and then click “Browse button”.

Select the firmware image file from the firmware package. The image filename extension is .ima. After that, click the “Start firmware update” button. It will immediately appear a new button “Proceed to Flash” and click it.

It pops a message window like below and click OK. Then, it will start to upload firmware ima file.

After uploading completed, it will appear two checkboxes. Select “Full flash” and click “Flash to Proceed”.

It will pop a window to confirm if you really want to update firmware, click OK.

Wait about 3 minutes, the following message shows and reset BMC then firmware update is finished.

## Remote update (Yafuflash)

1. Remote update BMC firmware by yafuflash.

a. Boot to Linux.

b. Enter BMC firmware directory [xxxxxxxyyzzzz];

xxxxxxx: project name ;

yy: system SKU number ;

zzzz: BMC firmware version.

c. Execute a.sh shell script to update BMC firmware.

Example:

Linux x64

#cd HA401VG012033/Linux

Linux #./yafuflash.sh -nw -ip [BMC IP] -u admin -p admin ../ HA401.ima

Note:

1. Make sure all the files can be executed, please use #chmod +x * in the update folder.

2. “-u admin -p admin” is the default user account and password, if you changed, need to apply the changed account and password.

3. The firmware name will be changed, please check the *.ima correct file name.

4. If you want to keep static IP, you need to add the parameter ‘-preserve-config’ , the instruction will be: ./yafuflash.sh -nw -ip [BMC IP] -u admin -p admin -preserve-config ../ HA401.ima

# SOC Flash update

When YafuFlash can’t work or have some problem, user can use SOCFlash force update firmware.

If user want to Full firmware update please execute SOCFlash.

Step1: Change directory to firmware directory and execute “A.bat soc”.

Dos

A:>cd HA401VG012033

A:\ HA401VG012033> A.bat soc

Linux

Linux x64

#cd HA401VG012033/Linux

Linux#./a.sh soc

Step2: After update BMC firmware, please restart system.

# Remote update BIOS firmware via BMC guide

## Yafuflash

a. Boot to Linux.

b. Enter BMC firmware directory [xxxxxxxyyzzzz];

xxxxxxx: project name ;

yy: system SKU number ;

zzzz: BMC firmware version.

c. Copy BIOS firmware file (*.bin) to Linux folder.

Example:

Linux x64

# cp bios.bin HA401VG012033 /Linux/

d. Shutdown the host or remote send power off system command.

# ipmitool -I lanplus -H <BMC_IP> -U [username] -P [password] power off

e. Execute update BIOS command

Linux x64

# cd HA401VG012033 /Linux/

# ./Yafuflash64 -nw -u [username] -p [password] -ip <BMC_IP> -d 2 bios.bin

PS. This is an example, the "HA401VG012033" is the BMC release package name, please replace the "HA401VG012033" with your BMC firmware package.

The “bios.bin” is the BIOS image name, please replace the "bios" with your BIOS image.

[username] and [password] are the same as IPMI.

Note:

- Please change your account to root, or you will get an exception failure.

## WEB GUI

This wizard takes you through the process of host BIOS firmware upgradation.

A screenshot of BIOS Firmware Update is as shown below.

To perform BIOS Firmware Update operation, click Maintenance BIOS Firmware Update from the menu bar. A sample screenshot is displayed below.

Procedure

- Click Browse to select BIOS Firmware image.

NOTE

Firmware update wizard will detect .bin extension as BIOS firmware image.

- Click Start Firmware Update to load the BIOS firmware image information. A sample screenshot is displayed below.

- Click Proceed, it will prompt you with the warning message. Click Ok to start the firmware update.

- The BIOS Firmware Update undergoes the below steps.

- Uploading Firmware Image

- Flashing Firmware Image

A sample screenshot is displayed below.

NOTE

You will not be able to perform any other tasks until firmware upgrade is completed and the device is off.

Once the BIOS firmware update is completed, it will prompt you with the success message. Click OK to complete the process. A sample screenshot is displayed below.