| Revision | ECN | Description | Date (yyyy/mm/dd) | Approval |
| --- | --- | --- | --- | --- |
| A |  | FIRST RELEASE | 2013/12/30 |  |
| B |  | Add SOCFlash update | 2015/11/30 |  |
| C |  | Modify execute command | 2016/03/29 |  |
| D |  | Modify WEB GUI screenshot | 2019/10/30 |  |
| E |  | - Remove DOS update - Add UEFI update | 2020/06/01 | Carter |
| F |  | - Add enable SOCFalsh by J48 Jumper instruction. - Remove Yafuflash update under x86 Linux OS. | 2021/04/20 | Mace |
| G |  | - Add BIOS firmware update via Yafuflash - Add BIOS firmware update via WEB GUI - Update BMC firmware package naming rule - Add notice for fan speed change when BMC is in FW update mode | 2022/02/10 | Sara |
| H |  | - Add remote update BMC by Yafuflash | 2022/04/28 | Sara |

1	Scope	3

2	Reference Docs	3

3	Requirement	4

4	Update guide	5

Notice: Fan speed will be set to 80% duty cycle when BMC is in FW update mode.	5

4.1	LINUX	5

4.2	Remote update (Yafuflash)	6

4.3	Remote update (WEB GUI)	6

4.4	SOC Flash update	11

4.5	UEFI update	11

5	Remote update BIOS firmware via BMC guide	12

5.1	Yafuflash	12

5.2	WEB GUI	12

# Scope

The scope of this document is to update BMC firmware.

# Reference Docs

# Requirement

Support OS & browser list.

LINUX:  Redhat 6.2, fedora 17, CentOS 6.2

UEFI:

Brower:

- I.E 6.0 or later version

- Firefox 24.0 or later version

- Chrome 31.0 or later version

# Update guide

## Notice: Fan speed will be set to 80% duty cycle when BMC is in FW update mode.

## LINUX

1. Update BMC firmware.

a. Boot to Linux.

b. Enter BMC firmware directory [xxxxxyy123456] ;

xxxxx: project name;

yy: mother board name;

12: Major Revision;

34: Minor Revision;

56: Auxiliary Revision.

c. Execute a.sh shell script to update BMC firmware.

Example:

Linux x64

#cd S1023TU010102/Linux

Linux#./a.sh

PS: This is just an example, "S1023TU010102" is BMC firmware file name.

Please instead "S1023TU010102" to your BMC firmware directory name.

d. If you don’t want to save config and want to use ”YafuFlash” to update, you can use “a.sh noconfig”

e. If you don’t want to save config, you can use “a.sh soc” to update.

f. If BMC is not responding, you can use “a.sh fix” to update.

PS: config will be clear.

2. After update BMC firmware, please power off and then power on system.

Note:

1. Please change your account to root, or you will get an exception failure.

2. In some critical conditions, after updating BMC firmware, you might need to unplug the AC power cord for 5 seconds and then plug in the AC power cord to reset the BMC, and then the updated new function can work properly.

3. For full firmware update, please execute “a.sh soc”.

## Remote update (Yafuflash)

1.	Remote Update BMC firmware

a.	Boot to remote Linux server

b.	Open Terminal

c.	Ping BMC IP. Make sure remote server can ping BMC.

d.	Enter in BMC firmware directory [xxxxxyy123456];

xxxxx: project name;

yy: motherboard name;

12: major revision;

34: minor revision;

56: auxiliary revision;

e.	Yafuflash update BMC command:

Update BMC flash with preserve config file.

# ./Yafuflash -nw -u [username] -p [password] -ip <BMC_IP> -fb -preserve-config ../rom.ima

f.	Update BMC flash without preserve config file.

# ./Yafuflash -nw -u [username] -p [password] -ip <BMC_IP> ../rom.ima

## Remote update (WEB GUI)

Log into the BMC web page

Click the “Maintenance”, and then click the “Firmware Update”

Select “Preserve all Configuration” to preserve all setting.

Or click “Edit Preserve Configuration” link to select the independent setting which you would like to preserve.

Return to Firmware Update page and then click “Firmware Update” link.

Select the firmware image file from the firmware package, the image filename extension is .ima, then click the “Start firmware update” button.

It will pop a dialog to confirm if you want to update firmware, please click OK.

Wait about 3 minutes, it shows the download procedure is finished, and then BMC will be rebooted.

## SOC Flash update

When YafuFlash can’t work or have some problems, users can use SOCFlash to force update firmware.

If users want to update full firmware, please execute SOCFlash.

Step1: Short J4 jumper, 1-2 to enable socflash.

Step2: Reset BMC.

Setp3: After BMC reboot completed, run socflash.

Step4:

Change directory to firmware directory and execute “./a.sh soc”.

Example:

Linux

Linux x64

#cd S1023TU010102/Linux

Linux#sudo ./a.sh soc

## UEFI update

Please execute flash.nsh

# Remote update BIOS firmware via BMC guide

## Yafuflash

Step1: Boot into linux OS by remote server.

Step2: Copy and Extract BMC firmware package into Download folder.

Step3: Enter BMC firmware directory [xxxxxyy123456]:.

xxxxx: project name;

yy: motherboard name;

12: major revision;

34: minor revision;

56: auxiliary revision;

Step4: Copy and Extract BIOS firmware package into Download folder.

Step5: Enter BIOS firmware folder to copy BIOS firmware file (*.bin).

# cp bios.bin ../S2013TU010102/Linux

Step6: Shutdown the host or send power off via remote ipmi command.

# ipmitool –I lanplus –H <BMC_IP> -U [username] –P [password] power off

Step7: Update BIOS via yafuflash command.

# cd S2013TU010102/Linux/

# ./Yafuflash –nw –u [username] –p [password] –ip <BMC_IP> -d 2 bios.bin

PS: This is just an example, "S2013TU010102" is BMC firmware file name.

Please instead "S2013TU010102" to your BMC firmware directory name.

And [username] and [password] are the same as IPMI.

Note:

1. Please change your account to root, or you will get an exception failure.

## WEB GUI

Step1: Log into the BMC web page.

Step2: Click the “Power Control” and select “Power off” to shutdown System.

Step3: Click the “Maintenance”, and then click the “Firmware Update”

Step3: “Select BIOS Firmware Image”

Step4: Click “Start Firmware update”.

Step5: Click “Proceed” to upload BIOS firmware.

Step6: Click “Proceed” to flash BIOS Firmware.

Step6: Wait for update complete.